11级单片机上机实验四

1.将R5和R6中的数据相乘，积的低8位放在50H中，高8位放在51H中。

2.在外部RAM首地址为1000H的数据表中连续存放着10个字节的数据，将每个字节的最高位和最低位置1，其它位不变。

3.用P0.7按键控制P1口跑马灯，按一下跑一位，同时用P2显示P1口亮灯的位号。

4.T0外部计数3个中断一次，中断处理中让p1口和p2口分别显示T0中断次数的信息。

实例1：p1口和p2口分别显示p0口按键按下的位信息

#include<at89x51.h>

void main (void)

{

code char smg[16]={0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,0x07,0x7f,0x6f,0x77,0x7c,0x39,0x5e,0x79,0x71};

char abc,xyz;

P1=0;

P2=0;

while(P0==0xff);

while(1)

{

switch(P0)

{

case 0xfe :abc=0;xyz=0x01;break;

case 0xfd :abc=1;xyz=0x02;break;

case 0xfb :abc=2;xyz=0x04;break;

case 0xf7 :abc=3;xyz=0x08;break;

case 0xef :abc=4;xyz=0x10;break;

case 0xdf :abc=5;xyz=0x20;break;

case 0xbf :abc=6;xyz=0x40;break;

case 0x7f :abc=7;xyz=0x80;break;

}

P2=smg[abc];

P1=xyz;

while(P0==0xff);

}

}

实例2：p1口和p2口分别显示外部中断源0中断次数的信息。

#include<reg51.h>

#define uchar  unsigned char

uchar code dispcode[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,0x07,0x7f,0x6f,0x77,0x7c,0x39,0x5e,0x79,0x71};

uchar abc=0;xyz=1;

void int_srv() interrupt 0

{

P1=xyz;

P2=dispcode[abc];

abc++;

xyz<<=1;

if(abc==8)

{

abc=0;

xyz=1;

}

//while(INT0==0);

}

void  main()

{

P1=0;

P2=0;

EA = 1;

EX0 = 1;

IT0=1;

while(1);

}